Troubleshooting

	Logger does not switch on
	· make sure no cable is connected to the USB port

· make sure the batteries are charged

· if this does not work proceed with instructions below

	Lock up: logger does not react on any user interaction
	· remove the elastic Velcro (pole mounting) with one screw at the back

· remove battery cover (screw on the bottom)

· remove the batteries for a few seconds

· insert batteries again, close cover and reattach Velcro

· restart logger

	Logger beeps twice: general alert
	· On start-up screen: enter QNH to start logging.

· Read instructions on the screen.

	Logger does not show a position 
	· It might take a few minutes to get a GPS fix.
· Make sure the logger is mounted in an upright position and has clear visibility of the sky.



[image: image1.jpg]



Balloon Competition Logger
 Guia rapida de utilización
Switch On
1. Apretar [image: image2.png]


para encender el  logger
USB cable must not be connected at that time.
2. Apretar [image: image3.png]


para confirmar
3. Entrar el QNH
Use the numbers from the task sheet.

4. Vuestro logger esta preparado
Idle Screen

	[image: image20.png]7 =
Pasition

5519761
32U 0314024

Atude

569t o 1013

150 27072009

s soL




Satellites / Battery status
	

	Position
	
[image: image4.png]




	Altitude (barometric)
	

	Time (local)
	

	Pilot ID, Logger S/N
	

	Markers dropped
	

	Goals declared
	

	Soft keys

Use first rows of keys to select
	


Switch Off

1. Apretar [image: image5.png]


 unos 5 secundos
2. Apretar [image: image6.png]


para confirmar
It takes approx. 15 seconds to write the track file to the SD Card.
Drop a Marker


1. Apretar [image: image7.png]Drop Marker



 I selecionar el testigo numero 
[image: image8.png]


 - [image: image9.png]


 para preparar el logger para drop marker .
2. Vuestro logger esta preparado para tirar el testigo. 
It still shows your position in the lower part of the screen.
3. Apretar [image: image10.png]


para tirar el testigo o [image: image11.png]


 para volver a la pantalla principal.
The relevant icon on the idle screen turns to black background for confirmation.
Declare Goal


1. Apretar [image: image12.png]Declare Goal



 I selecionar la meta  [image: image13.png]


 - [image: image14.png]


 para preparar el logger para la declaracion de la meta.
2. Entrar la declaracion de vuestra meta (3 digitos). Utiliza flechas [image: image15.png]Drop Marker



 [image: image16.png]Declare Goal



 para mover el  cursor.
3. Entrar la altitud si necesario o apretar [image: image17.png]Declare Goal




4. Apretar [image: image18.png]


para salvar la declaración o [image: image19.png]


 para volver a la pantalla principal.

Sensitive area





Sensitive area








